CpE 3105: Computer Networks and Security

Skills Assessment 2

Instructions

In this assessment, you will create a small network shown in the topology below, using your Packet Tracer and configure its devices. Configure the routers, switches and PCs to support IPv4 connectivity. Use the following resources: 1 Cisco 2911 Router, 3 Cisco 2960 Switches and 3 PCs. Follow the connections and its ports as shown in the topology.

Topology

IPv4 Addressing Scheme

Given the network address shown below, design an IP addressing scheme that satisfies the requirement.

| Network | Number of Hosts in LAN A | Number of Hosts in LAN B | Number of Hosts in LAN C |
| --- | --- | --- | --- |
| 10.0.0.0/8 | 5 | 100 | 50 |

The host computers will use the LAST available IP address in the given network. Switches will use the second to the last available host address. Ports on the network router will use the FIRST available host address. Write down the IP address information for each device: (use red font)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | Gig 0/0 | 10.0.0.193 | 255.255.255.248 | NA |
|  | Gig 0/1 | 10.0.0.129 | 255.255.255.192 | NA |
|  | Gig 0/2 | 10.0.0.1 | 255.255.255.128 | NA |
| SW-Floor1 | VLAN 1 | 10.0.0.197 | 255.255.255.248 | 10.0.0.193 |
| SW-Floor2 | VLAN 1 | 10.0.0.125 | 255.255.255.128 | 10.0.0.1 |
| SW-Floor3 | VLAN 1 | 10.0.0.189 | 255.255.255.192 | 10.0.0.129 |
| PC-A |  | 10.0.0.198 | 255.255.255.248 | 10.0.0.193 |
| PC-B |  | 10.0.0.126 | 255.255.255.128 | 10.0.0.1 |
| PC-C |  | 10.0.0.190 | 255.255.255.192 | 10.0.0.129 |

Configure Device IPv4 and Security Settings

- Configure the host computers. Use the IP addresses you provided on the address table above.

- Configure HQ. Configuration tasks for HQ include the following:

  - Router Name: HQ

  - Encrypted privileged EXEC password: cpeenpass

  - Console access password: cpeconpass

  - Telnet access password: admin1pass

  - Encrypt the clear text passwords

  - Provide the message of the day: You are in HQ router.

  - Interface Gig 0/0: Set the layer 3 IPv4 Address, Set a description, and activate interface

  - Interface Gig 0/1: Set the layer 3 IPv4 Address, Set a description, and activate interface

  - Interface Gig 0/2: Set the layer 3 IPv4 Address, Set a description, and activate interface

- Configure the switches. Configuration tasks include the following:

  - Switch names. Refer to the address table above.

  - Set the Layer 3 IPv4 address on the vlan1 interface of each switch. Provide the default gateway as well.

  - Encrypted privileged EXEC password: cpeenpass

  - Console access password: cpeconpass

  - Telnet access password: admin1pass

  - Encrypt the clear text passwords

  - Provide the message of the day: Welcome to <hostname> switch!

- Verify the network connectivity. Use the ping command to test the connectivity between all network devices. Use the following table to methodically verify connectivity with each network device:

| From | To | IP Address | Ping Results (success,fail) |
| --- | --- | --- | --- |
| PC-A | HQ, G0/0 |  |  |
| PC-A | HQ, G0/1 |  |  |
| PC-A | HQ, G0/2 |  |  |
| PC-A | SW-Floor1 |  |  |
| PC-A | SW-Floor2 |  |  |
| PC-A | SW-Floor3 |  |  |
| PC-A | PC-B |  |  |
| PC-A | PC-C |  |  |
| PC-B | HQ, G0/0 |  |  |
| PC-B | HQ, G0/1 |  |  |
| PC-B | HQ, G0/2 |  |  |
| PC-B | SW-Floor1 |  |  |
| PC-B | SW-Floor2 |  |  |
| PC-B | SW-Floor3 |  |  |
| PC-B | PC-A |  |  |
| PC-B | PC-C |  |  |
| PC-C | HQ, G0/0 |  |  |
| PC-C | HQ, G0/1 |  |  |
| PC-C | HQ, G0/2 |  |  |
| PC-C | SW-Floor1 |  |  |
| PC-C | SW-Floor2 |  |  |
| PC-C | SW-Floor3 |  |  |
| PC-C | PC-A |  |  |
| PC-C | PC-C |  |  |

- Save your packet tracer file as SKA2.pkt and submit it together with this document (saved as SKA2.pdf) in your Canvas.

enable

configure terminal

hostname HQ

enable secret cpeenpass

service password-encryption

banner motd "You are in HQ router."

line console 0

password cpeconpass

login

exit

line vty 0 4

password admin1pass

login

exit

interface gigabitEthernet 0/0

description Link to LAN A

ip address 10.0.0.193 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/1

description Link to LAN C

ip address 10.0.0.129 255.255.255.192

no shutdown

exit

interface gigabitEthernet 0/2

description Link to LAN B

ip address 10.0.0.1 255.255.255.128

no shutdown

end

Copy run start

enable

configure terminal

hostname SW-Floor1

enable secret cpeenpass

service password-encryption

banner motd "Welcome to SW-Floor1 switch!"

line console 0

password cpeconpass

login

exit

line vty 0 4

password admin1pass

login

exit

! IP Config

interface vlan 1

ip address 10.0.0.197 255.255.255.248

no shutdown

exit

ip default-gateway 10.0.0.193

end

Copy run start

enable

configure terminal

hostname SW-Floor2

enable secret cpeenpass

service password-encryption

banner motd "Welcome to SW-Floor2 switch!"

line console 0

password cpeconpass

login

exit

line vty 0 4

password admin1pass

login

exit

! IP Config

interface vlan 1

ip address 10.0.0.125 255.255.255.128

no shutdown

exit

ip default-gateway 10.0.0.1

end

Copy run start

enable

configure terminal

hostname SW-Floor3

enable secret cpeenpass

service password-encryption

banner motd "Welcome to SW-Floor3 switch!"

line console 0

password cpeconpass

login

exit

line vty 0 4

password admin1pass

login

exit

! IP Config

interface vlan 1

ip address 10.0.0.189 255.255.255.192

no shutdown

exit

ip default-gateway 10.0.0.129

end

write memory